WRONG PART INSTALLED DANGEROUS / HAZARDOUS

The motor they installed is 100% NOT a condenser fan motor.

Measurements:

- 11W

- 0.19A

- 2850 RPM

- Shaft length: 1.6 inches

Matches EXACTLY the characteristics of generic aftermarket EVAPORATOR MOTORS, not condenser motors.

And they do NOT match the required GE condenser motor (WR60X10168), whose specs are:

- 11W

- 0.193A

- 2200 RPM

- Shaft ≈ 1.75 inches

This means the part they installed:

❌ Spins too fast

❌ Has incorrect torque

❌ Is sized incorrectly

❌ Does not match GE blade profile

❌ Hit the sides of the cooling shroud

❌ Fan blades fall off

❌ Is not heat-rated for condenser area

❌ Is the wrong motor class entirely

And the WR00X3041 part number you gave earlier is VERIFIED as an evaporator fan motor, NOT a condenser motor.

Why this is dangerous and damaging

1️⃣ 2850 RPM is WRONG for the condenser

The condenser blade is designed for ~2200 RPM.
2850 RPM causes:

- airflow imbalance

- vibration

- bearing stress

- poor heat dissipation

- motor overheating

- compressor overheating

2️⃣ Evaporator motors aren’t designed for HOT environments

Condenser compartments routinely exceed:

- 120°F–160°F

Evaporator motors are built for:

- freezer temperatures

They WILL overheat and fail — or melt wiring.

3️⃣ It threatens the compressor

The compressor already hit 140°F, which is dangerously high.