If you decide to attempt to repair the 20-year-old refrigerator one more time, then these must be done:

Technician Checklist (Refrigerator Condenser Fan Area)

□ Install OEM or manufacturer‑approved condenser fan motor (brand/model recorded).

Brand: ____________________ Model #: ______________________

□ Use factory connector/harness—do not cut; remove non‑factory crimp splices.

□ Verify fan rotation and airflow: across condenser & over compressor, exhausting out grille (NOT AWAY FROM THE CONDENSER).

□ Install new condenser fan blade with correct hub/bore; fully seated; no shroud rub.

□ Bench test motor (blade off): immediate start; continuous run; no thermal trip.

□ 15–20 min run test: steady airflow; condenser no longer overheating.

□ Cooling verification: fresh‑food section trending to ≤ 39 °F and holding.

Tech (print):  ________________________________

Sign: ________________________________ Date: ____________